# ROS 2 Workspace Setup Commands

This document outlines the commands used to create the ROS 2 workspace, initialize the package, build it, and run the pipeline.

## Command:

source /opt/ros/jazzy/setup.bash

### Explanation:

Sources the core ROS 2 Jazzy underlay, making the 'ros2' and 'colcon' tools available in the current terminal session.

## Command:

mkdir -p ros2_ws/src
cd ros2_ws/src

### Explanation:

Creates the standard ROS 2 workspace directory structure and navigates into the source folder where packages will live.

## Command:

ros2 pkg create --build-type ament_python leap_deployment --dependencies rclpy sensor_msgs std_msgs

### Explanation:

Generates a new Python-based ROS 2 package named 'leap_deployment'. It also configures the package.xml to declare dependencies on standard ROS 2 message libraries.

## Command:

mkdir -p leap_deployment/launch leap_deployment/urdf leap_deployment/rviz

### Explanation:

Creates the necessary subdirectories inside the package to store our custom launch scripts, the robot URDF model, and the RViz visualization configurations.

## Command:

cd ..
colcon build --symlink-install

### Explanation:

Compiles the entire ROS 2 workspace. The '--symlink-install' flag is used so that changes made to Python scripts automatically apply without needing to run 'colcon build' again.

## Command:

source install/setup.bash

### Explanation:

Sources the newly built workspace overlay, telling the ROS 2 environment exactly where to find our new 'leap_deployment' package and its executables.

## Command:

ros2 launch leap_deployment display.launch.py

### Explanation:

Executes the launch file, which automatically starts the robot_state_publisher, policy_node, interface_node, and opens RViz2 for visualization simultaneously.