# Task B: Policy Deployment via ROS 2

RRC IIIT-H Research Internship  |  Optional  |  Deadline: May 12, 2026

## Objective

Bridge simulation and real-world deployment by setting up a ROS 2 pipeline. Run the trained Task A policy in inference mode and stream joint commands to visualize the LEAP Hand motion in RViz2.

## Stack

- ROS 2 Humble

- Python (ament_python build type)

- sensor_msgs/JointState for RViz2 animation

- Stable Baselines 3 PPO.load() for policy inference

- RViz2 for visualization

## Key Technical Notes

Node 1 - policy_node.py: Loads SB3 checkpoint, publishes 16 joint targets to /hand/joint_commands as Float32MultiArray at 10 Hz

Node 2 - interface_node.py: Subscribes to /hand/joint_commands, converts to sensor_msgs/JointState with correct URDF joint names, publishes to /joint_states

Node 3 - robot_state_publisher: Reads /joint_states and broadcasts TF transforms so RViz2 can render the hand

Fallback: If Task A policy did not converge, replace policy_node with a sine wave generator. This is explicitly allowed and still demonstrates full ROS integration skill

Topic flow: /hand/joint_commands (Float32MultiArray) -> interface_node -> /joint_states (JointState) -> RViz2

## Day-wise Task Prompts

Paste each row's prompt into Antigravity > Agent Manager > New Task on the given date. One task per day. Do not feed future tasks early.

| Date | Day | Focus Area | Prompt | Expected Deliverable |
| --- | --- | --- | --- | --- |
| May 7 | Day 1 | ROS 2 Workspace Setup | Create a ROS 2 (Humble) workspace: ros2_ws/src/. Inside src/, create a package named leap_hand_deploy using: ros2 pkg create --build-type ament_python leap_hand_deploy. Add dependencies: rclpy, sensor_msgs, std_msgs in package.xml. Build with colcon build and source install/setup.bash. Verify package is found with ros2 pkg list. | Workspace builds without errors. Package visible in ros2 pkg list. |
| May 8 | Day 2 | URDF + Launch File | Copy right_hand.xml from mujoco_menagerie and reference the URDF. Write a launch file launch/display.launch.py that starts robot_state_publisher with the URDF and opens RViz2. Verify the hand skeleton appears in RViz2 with no transform errors. | Hand visible in RViz2. No TF errors in terminal. |
| May 9 | Day 3 | Policy Inference Node | Write nodes/policy_node.py. Load the best Task A checkpoint using SB3's PPO.load(). On a timer at 10 Hz, run model.predict() with a dummy or zero observation. Publish the 16 joint targets to topic /hand/joint_commands as a Float32MultiArray. Verify topic is publishing with: ros2 topic echo /hand/joint_commands. If Task A policy did not converge, substitute with a sine wave that generates 16 values cycling between joint limits. | /hand/joint_commands publishing at 10 Hz. Values are valid floats. Fallback sine wave branch created if needed. |
| May 10 | Day 4 | Interface Node | Write nodes/interface_node.py. Subscribe to /hand/joint_commands. Convert the 16 float targets into a sensor_msgs/JointState message with correct joint names matching the URDF. Publish to /joint_states at 10 Hz. Verify RViz2 animates the hand in response to published values. | Hand moves in RViz2 driven by policy or sine wave output. /joint_states visible with ros2 topic echo. |
| May 11 | Day 5 | Integration + Recording | Write a single launch file launch/full_pipeline.launch.py that starts all four nodes: policy_node, interface_node, robot_state_publisher, RViz2. Run the full pipeline and verify with ros2 node list that all nodes are active. Record a screen capture showing the node list and the hand moving in RViz2. Save as videos/ros2_demo.mp4. | ros2_demo.mp4 saved. All 4 nodes visible in ros2 node list. Full pipeline launches with one command. |
| May 12 | Day 6 | Documentation + Submission | Write the Task B section of README.md covering: ROS node architecture (text diagram is fine), build instructions (colcon build), launch command (ros2 launch leap_hand_deploy full_pipeline.launch.py), and topic diagram showing /hand/joint_commands and /joint_states. Push Task B code to its own GitHub repo. Submit both repo links, videos, and PDF report on the Google Form before the deadline. | GitHub repo pushed. Form submitted before deadline. README covers all nodes and topics. |